# PingPair — LAN Characterization Report

| Run ID | PingPair-sample-report |
| --- | --- |
| Started | 2026-06-09 14:30:00 |
| Finished | 2026-06-09 14:45:50 |
| Duration | 15m 50s |
| Server IP | 192.168.1.1 |
| Client IP | 192.168.1.2 |
| Protocol | UDP |
| Cases ok | 20 / 20 |
| fping version | 5.5 |
| iperf3 version | 3.21 |
| PingPair version | 0.1.0 |
| Technician | Demo User |
| Customer / Site | Example Co. (sample report) |
| Test environment | Two laptops · Cat6 · point-to-point |
| Test record ID | SAMPLE-001 |
| Cable length (m) | 15.0 |

# Performance Metrics

Steady-state network with no external simulated or video traffic. QoS: COS set to 0.

| Payload Size (Bytes) | Bandwidth (MBits Per Second) – Pushed Traffic | Throughput (MBits Per Second) – Received Traffic | Jitter (msec) | Packet Loss (%) | Minimum Latency (msec) | Average Latency (msec) | Maximum Latency (msec) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 200 | 10 | 9.90 | 0.105 | 0.000 | 0.13 | 0.96 | 2.60 |
| 200 | 30 | 29.70 | 0.235 | 0.000 | 0.15 | 1.08 | 3.11 |
| 200 | 50 | 49.20 | 0.365 | 0.070 | 0.17 | 1.20 | 3.62 |
| 200 | 70 | 68.00 | 0.495 | 0.540 | 0.18 | 1.32 | 4.13 |
| 200 | 90 | 85.50 | 0.625 | 1.770 | 0.20 | 1.44 | 4.64 |
| 600 | 10 | 10.00 | 0.125 | 0.000 | 0.13 | 1.01 | 2.73 |
| 600 | 30 | 29.80 | 0.255 | 0.000 | 0.15 | 1.13 | 3.24 |
| 600 | 50 | 49.50 | 0.385 | 0.060 | 0.17 | 1.25 | 3.75 |
| 600 | 70 | 68.30 | 0.515 | 0.500 | 0.18 | 1.37 | 4.26 |
| 600 | 90 | 85.90 | 0.645 | 1.610 | 0.20 | 1.49 | 4.77 |
| 1000 | 10 | 10.00 | 0.145 | 0.000 | 0.13 | 1.06 | 2.86 |
| 1000 | 30 | 29.90 | 0.275 | 0.000 | 0.15 | 1.18 | 3.37 |
| 1000 | 50 | 49.60 | 0.405 | 0.060 | 0.17 | 1.30 | 3.88 |
| 1000 | 70 | 68.60 | 0.535 | 0.450 | 0.18 | 1.42 | 4.39 |
| 1000 | 90 | 86.20 | 0.665 | 1.460 | 0.20 | 1.54 | 4.90 |
| 1300 | 10 | 10.00 | 0.165 | 0.000 | 0.13 | 1.11 | 2.99 |
| 1300 | 30 | 30.00 | 0.295 | 0.000 | 0.15 | 1.23 | 3.50 |
| 1300 | 50 | 49.90 | 0.425 | 0.050 | 0.17 | 1.35 | 4.01 |
| 1300 | 70 | 68.90 | 0.555 | 0.400 | 0.18 | 1.47 | 4.52 |
| 1300 | 90 | 86.60 | 0.685 | 1.300 | 0.20 | 1.59 | 5.03 |

# Per-case detail

Forensics for each case: status, error notes if any, subprocess return codes, and the exact CLI strings used. Useful for re-running a single case from a terminal.

## Case #01 — payload 200 B, bandwidth 10 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 9.900 Mbps |
| Jitter | 0.105 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.13 / 0.96 / 2.60 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 200 -b 10M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #02 — payload 200 B, bandwidth 30 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 29.700 Mbps |
| Jitter | 0.235 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.15 / 1.08 / 3.11 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 200 -b 30M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #03 — payload 200 B, bandwidth 50 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 49.200 Mbps |
| Jitter | 0.365 ms |
| Loss | 0.070 % |
| Min / Avg / Max latency | 0.17 / 1.20 / 3.62 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 200 -b 50M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #04 — payload 200 B, bandwidth 70 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 68.000 Mbps |
| Jitter | 0.495 ms |
| Loss | 0.540 % |
| Min / Avg / Max latency | 0.18 / 1.32 / 4.13 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 200 -b 70M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #05 — payload 200 B, bandwidth 90 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 85.500 Mbps |
| Jitter | 0.625 ms |
| Loss | 1.770 % |
| Min / Avg / Max latency | 0.20 / 1.44 / 4.64 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 200 -b 90M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #06 — payload 600 B, bandwidth 10 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 10.000 Mbps |
| Jitter | 0.125 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.13 / 1.01 / 2.73 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 600 -b 10M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #07 — payload 600 B, bandwidth 30 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 29.800 Mbps |
| Jitter | 0.255 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.15 / 1.13 / 3.24 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 600 -b 30M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #08 — payload 600 B, bandwidth 50 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 49.500 Mbps |
| Jitter | 0.385 ms |
| Loss | 0.060 % |
| Min / Avg / Max latency | 0.17 / 1.25 / 3.75 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 600 -b 50M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #09 — payload 600 B, bandwidth 70 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 68.300 Mbps |
| Jitter | 0.515 ms |
| Loss | 0.500 % |
| Min / Avg / Max latency | 0.18 / 1.37 / 4.26 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 600 -b 70M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #10 — payload 600 B, bandwidth 90 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 85.900 Mbps |
| Jitter | 0.645 ms |
| Loss | 1.610 % |
| Min / Avg / Max latency | 0.20 / 1.49 / 4.77 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 600 -b 90M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #11 — payload 1000 B, bandwidth 10 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 10.000 Mbps |
| Jitter | 0.145 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.13 / 1.06 / 2.86 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1000 -b 10M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #12 — payload 1000 B, bandwidth 30 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 29.900 Mbps |
| Jitter | 0.275 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.15 / 1.18 / 3.37 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1000 -b 30M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #13 — payload 1000 B, bandwidth 50 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 49.600 Mbps |
| Jitter | 0.405 ms |
| Loss | 0.060 % |
| Min / Avg / Max latency | 0.17 / 1.30 / 3.88 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1000 -b 50M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #14 — payload 1000 B, bandwidth 70 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 68.600 Mbps |
| Jitter | 0.535 ms |
| Loss | 0.450 % |
| Min / Avg / Max latency | 0.18 / 1.42 / 4.39 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1000 -b 70M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #15 — payload 1000 B, bandwidth 90 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 86.200 Mbps |
| Jitter | 0.665 ms |
| Loss | 1.460 % |
| Min / Avg / Max latency | 0.20 / 1.54 / 4.90 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1000 -b 90M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #16 — payload 1300 B, bandwidth 10 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 10.000 Mbps |
| Jitter | 0.165 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.13 / 1.11 / 2.99 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1300 -b 10M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #17 — payload 1300 B, bandwidth 30 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 30.000 Mbps |
| Jitter | 0.295 ms |
| Loss | 0.000 % |
| Min / Avg / Max latency | 0.15 / 1.23 / 3.50 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1300 -b 30M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #18 — payload 1300 B, bandwidth 50 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 49.900 Mbps |
| Jitter | 0.425 ms |
| Loss | 0.050 % |
| Min / Avg / Max latency | 0.17 / 1.35 / 4.01 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1300 -b 50M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #19 — payload 1300 B, bandwidth 70 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 68.900 Mbps |
| Jitter | 0.555 ms |
| Loss | 0.400 % |
| Min / Avg / Max latency | 0.18 / 1.47 / 4.52 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1300 -b 70M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

## Case #20 — payload 1300 B, bandwidth 90 Mbps (30 s, UDP)

| Status | ok |
| --- | --- |
| Error | — |
| iperf3 client rc | 0 |
| iperf3 server rc | — |
| fping rc | 0 |
| Throughput received | 86.600 Mbps |
| Jitter | 0.685 ms |
| Loss | 1.300 % |
| Min / Avg / Max latency | 0.20 / 1.59 / 5.03 ms |
| iperf3 client cmd | iperf3.exe -c 192.168.1.1 -B 192.168.1.2 -u -i 1 -t 30 -l 1300 -b 90M |
| iperf3 server cmd | iperf3.exe -s -i 1 -B 192.168.1.1 -1 --json |
| fping cmd | fping.exe 192.168.1.1 -S 192.168.1.2 -p 10 -c 3000 -s -D |

Generated by PingPair 0.1.0

# Analysis appendix

Per-metric summary statistics and per-case charts for this run, plus per-payload and per-bandwidth breakdowns. Open the Analysis tab in PingPair to overlay this run against past sweeps.

## Summary statistics

| Metric | Samples | Min | Avg | Median | Max | Stdev |
| --- | --- | --- | --- | --- | --- | --- |
| Throughput (Mbps) | 20 | 9.90 | 48.77 | 49.55 | 86.60 | 26.99 |
| Avg latency (ms) | 20 | 0.96 | 1.27 | 1.27 | 1.59 | 0.18 |
| Packet loss (%) | 20 | 0.000 | 0.414 | 0.060 | 1.770 | 0.594 |
| Jitter (ms) | 20 | 0.105 | 0.395 | 0.395 | 0.685 | 0.185 |

## Per-case charts

### Throughput

### Avg latency

### Packet loss

### Jitter

## Mean metrics by payload size

| Bucket | Cases | Avg Throughput (Mbps) | Avg Avg (ms) | Avg Packet (%) | Avg Jitter (ms) |
| --- | --- | --- | --- | --- | --- |
| 200 B | 5 | 48.46 | 1.20 | 0.476 | 0.365 |
| 600 B | 5 | 48.70 | 1.25 | 0.434 | 0.385 |
| 1000 B | 5 | 48.86 | 1.30 | 0.394 | 0.405 |
| 1300 B | 5 | 49.08 | 1.35 | 0.350 | 0.425 |

### Avg throughput by payload

## Mean metrics by bandwidth

| Bucket | Cases | Avg Throughput (Mbps) | Avg Avg (ms) | Avg Packet (%) | Avg Jitter (ms) |
| --- | --- | --- | --- | --- | --- |
| 10 Mbps | 4 | 9.97 | 1.04 | 0.000 | 0.135 |
| 30 Mbps | 4 | 29.85 | 1.16 | 0.000 | 0.265 |
| 50 Mbps | 4 | 49.55 | 1.27 | 0.060 | 0.395 |
| 70 Mbps | 4 | 68.45 | 1.40 | 0.473 | 0.525 |
| 90 Mbps | 4 | 86.05 | 1.52 | 1.535 | 0.655 |

### Avg throughput by bandwidth